### ANTI SALT

# CHLORIDE AND SULPHATE SALT NEUTRALISER

A LOW VISCOSITY, SOLVENT FREE SALT NEUTRALISER

TARGETING CHLORIDES AND SULPHATES.

## DESCRIPTION AND FEATURES

ANTI SALT is a Chloride and sulphate salt neutraliser that converts water-soluble salts into insoluble or hardly soluble reactive compounds. As a result of this neutralisation, Chlorides and Sulphates, which are present in masonry, cannot be transported by the gauging solution used in slurry coat of tanking seal or to the surface of masonry. This in turn prevents salts transferring into tanking slurry when tanking below ground.

## PRODUCT DATA

| BASE | COLOUR | DENSITY | APPLICATION |
| --- | --- | --- | --- |
| ZINC FLOROSILICATE | CLEAR | APPROX 1.12Kg/dm3 | BRUSH/SPRAY |

## INSTRUCTION FOR USE

ANTI SALT is supplied as a concentrate for dilution with water prior to use. Apply to the prepared substrate in two operations as follows:

1st operation

Dilute one part of ANTI SALT with two parts water and apply.

2nd operation

Allow a minimum of three hours following the first operation and then apply one part of ANTI SALT, which has been diluted with one part of water.

#### COVERAGE

Approximately 500ml will cover 1sq metre for two operations.

## STORAGE

Keep container closed. Protect from frost. Once open the shelf life will be 12 months.

Read health and safety data sheet before use

Update 20.10.2010

Units 1 & 2a Lazarus Court

Woodgate - Rothley

Leicester - LE7 7NR

TEL: 0116 216 5354

www.constructionchemicals.co.uk