HEALTH & SAFETY DATA SHEET

- IDENTIFICATION OF THE SUBSTANCE & OF THE COMPANY UNDERTAKING

Product Name:			OXALIC ACID

Company:			Construction Chemicals Ltd

Units 1 & 2a, Lazarus Court, Woodgate, Rothley, LE7 7NR

Tel:					0116 216 5354

- COMPOSITION/INFORMATION ON INGREDIENTS

Ingredient			CAS No		Percent		Hazards

Oxalic Acid			144-	62-7		99-100%	YES

- HAZARDS IDENTIFICATION

Emergency overview

POISON! DANGER! MAY BE FATAL IF SWALLOWED. CORROSIVE, CAUSES SEVERE IRRITATION AND BURNS TO SKIN, EYES AND RESPIRATORY TRACT. HARMFUL IF INHALED OR ASORBED THROUGH SKIN. MAY CAUSE KIDNEY DAMAGE.

Potential Health Effects

Oxalic Acid is corrosive to tissue. When ingested, Oxalic Acid removes calcium from the blood. Kidney damage can be expected as the calcium is removed from the blood in the form of calcium oxalate. The calcium oxalate then obstructs the kidney tubules.

Inhalation: Toxic! May cause burns, nausea, severe gastroenteritis and vomiting, shock and convulsions. May cause renal damage, as evidenced by blood urine. Estimated fatal dose is 5 to 15 grams.

Skin contact: Can cause severe irritation, possible skin burns. May be absorbed through the skin.

Eye contact: Oxalic Acid is an eye irritant. It may produce corrosive effects.

Chronic Exposure:  May cause inflammation of the upper respiratory tract. Prolonged skin contact can cause dermatitis, cyanosis of the fingers and possible ulceration. May affect kidneys.

Aggravation of pre-existing conditions: Persons with pre—existing skin disorders or eye problems or impaired kidney or respiratory function may be more susceptible to the effects of the substance.

- FIRST AID MEASURE

Inhalation: Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician immediately.

Ingestion: DO NOT INDUCE VOMITING! Give large quantities of limewater or milk to drink. Never give anything by mouth to an unconscious person. Call a physician immediately.

Skin contact: In case of contact, wipe off excess from skin then immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before re-use. Call a physician immediately.

Eye contact: Immediately flush eyes with gentle but large stream of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Call a physician immediately.

5.	FIRE FIGHTING MEASURES

Fire: Oxalic Acid is a combustible solid below 101°C (215F)

Explosion: Reacts explosively with strong oxidizing materials and some silver compounds.

Fire Extinguishing Media: Water spray, dry chemical, alcohol foam or carbon dioxide. Foam or water on molten oxalic acid may cause frothing. Water spray may be used to keep fire exposed containers cool.

Special Information: In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full face piece operated in the pressure demand or other positive pressure mode.

6.	ACCIDENTAL RELEASE MEASURES

Remove all sources of ignition. Ventilation area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8.

Spills: Clean up spills in a manner that does not disperse dust into the air. Use non-sparking tools and equipment. Pick up spill for recovery or disposal and place in a closed container. Remove unnecessary people. If material comes in contact with water, neutralize liquid with alkaline (soda ash, lime) then absorb with an inert material (e.g. vermiculite, dry sand, earth) and place in a chemical waste container. Do no use combustible materials such as saw dust. Do not flush to sewer.

7.	HANDLING AND STORAGE

Keep in a tightly closed container. Protect from physical damage. Store in a cool, dry, ventilated area away from sources of heat, moisture and incompatibilities. Containers of this material may be hazardous when empty since they retain product residues (dust, solids); observe all warnings and precautions listed for the product.

8. 	EXPOSURE CONTROLS/PERSONAL PROTECTION

Airborne Exposure Limits: - ACGIH Threshold Limit Value (TLV): 1 mg/m3 (TWA), 2mg/m3 (STEL)

OSHA Permissible Exposure Limit (PEL): 1mg/m3

Ventilation system: A system of local and/or general exhaust is recommended to keep employees exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant as its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details.

Personal Respirators (NIOSH Approved): If the exposure limit is exceeded, a half-face respirator with an organic vapour cartridge and dust/mist filter may be worn for up to ten times the exposure limit or the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is the lowest. A full-face piece respirator with an organic vapour cartridge and dust/mist filter may be worn up to 50 times the exposure limit, or the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is the lowest. For emergencies or instances where the exposure levels are not known. Use a full-face piece positive-pressure, air-supplied respirator, WARNING – Air purifying respirators do not protect workers in oxygen deficient atmospheres.

Eye protection – Use chemical safety goggles and /or full face shield where dusting or splashing of solutions is possible. Maintain eye wash fountain and quick-drench facilities in work area.

Skin protection –.Wear impervious protective clothing including boots, gloves, lab coat, apron or coveralls as appropriate to prevent skin contact.

9.	PHYSICAL AND CHEMICAL PROPERTIES

Appearance:					Transparent. Colourless crystals

Odour:						Odourless

Solubility:					ca. 1g/7mL of water

Specific gravity:					1.65 @ 18.5 C/ 4C

Ph:						No information available

% volatiles by volume: 21C (70F):		0

Boiling point/boiling point range:			149 – 160C (300 – 320F) sublimes

Melting point/melting point range:		101.5C (216F)

Vapour density (Air=1):				4.4

Vapour pressure (mm Hg):			<0.001 @ 20C (68F)

Evaporation rate (BuAc=1):			No information found

10.	STABILITY AND REACTIVITY

Stability – Stable under ordinary conditions of use and storage. Heat will contribute to instability.

Hazard Decomposition Products – Carbon dioxide and carbon monoxide may form when heated to decomposition. May also form formic acid.

Hazardous Polymerization – Will not occur.

Incompatibility – Alkalis, chlorites, hypochlorites, oxidising agents, furfuryl alcohol and silver compounds.

Conditions to Avoid – Heat, ignition sources and incompatibilities.

11.	TOXICOLOGICAL INFORMATION

Oral rate LD50: 375mg/kg; irritation skin rabbit: 500mg/24H mild, eye rabbit 250ug/24H; investigated as a reproductive effector.

- ECOLOGICAL INFORMATION

Environmental Fate – No information found.

Environmental Toxicity – No information found.

- DISPOSAL RECOMMENDATIONS

Whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved waste facility. Although not a listed RCRA hazardous waste, this material may exhibit one or more characteristics of a hazardous waste and require appropriate analysis to determine specific disposal requirements. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

- TRANSPORT INFORMATION

Proper Shipping Name _ CORROSIVE, SOLID, ACIDIC, ORGANIC N.O.S. (OXALIC ACID, DIHYDRATE).

Hazard class: 8	UN No: 3261		Packing Group III

- REGULATORY INFORMATION

Chemical Inventory Status – Part 1

Ingredient			TSCA		EC		Japan		Australia

Oxalic Acid (144-62-7)		Yes		Yes		Yes		Yes

- OTHER INFORMATION

NFPA Ratings: Health: 3	Flammability: 1	Reactivity: 0

Label Hazard Warnings – POISON! DANGER! MAY BE FATAL IF SWALLOWED. CORROSIVE. CAUSES SEVERE IRRITATION AND BURNS TO SKIN, EYES AND REPIRATORY TRACT. HARMFUL IF INHALED OR ABSORBED THROUGH SKIN. MAY CAUSE KIDNEY DAMAGE.

Label Precautions: Do not breathe dust.

Do not get in eyes, on skin, or on clothing.

Keep container closed.

Use only with adequate ventilation.

Wash thoroughly after handling.

Label First Aid: If swallowed DO NOT INDUCE VOMITING! Give large quantities of lime water or milk to drink. Never give anything by mouth to an unconscious person. If inhaled, remove to fresh air. If not breathing give artificial respiration. If breathing is difficult give oxygen. In case of contact, wipe off excess material from skin then immediately flush eyes or skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. In all cases call a physician immediately.

Product use – Laboratory re-agent.

Revision information – no changes.